Complete, Dual, 12-/14-/16-Bit, Serial Input,

Unipolar/Bipolar, Voltage Output DACs

Data Sheet AD5722/AD5732/AD5752

FEATURES

Complete, dual, 12-/14-/16-bit digital-to-analog converter (DAC)  Operates from single/dual supplies

Software programmable output range

+5 V, +10 V, +10.8 V, ±5 V, ±10 V, ±10.8 V

INL error: ±16 LSB maximum, DNL error: ±1 LSB maximum  Total unadjusted error (TUE): 0.1% FSR maximum  Settling time: 10 μs typical

Integrated reference buffers

Output control during power-up/brownout

Simultaneous updating via LDAC

Asynchronous CLR to zero scale or midscale

DSP-/microcontroller-compatible serial interface  24-lead TSSOP

Operating temperature range: −40°C to +85°C  iCMOS process technology1

APPLICATIONS

Industrial automation

Closed-loop servo control, process control

Automotive test and measurement

Programmable logic controllers

GENERAL DESCRIPTION

The AD5722/AD5732/AD5752 are dual, 12-/14-/16-bit, serial  input, voltage output, digital-to-analog converters. They operate  from single-supply voltages from +4.5 V up to +16.5 V or dual supply voltages from ±4.5 V up to ±16.5 V. Nominal full-scale  output range is software-selectable from +5 V, +10 V, +10.8 V,  ±5 V, ±10 V, or ±10.8 V. Integrated output amplifiers, reference  buffers, and proprietary power-up/power-down control circuitry  are also provided.

The parts offer guaranteed monotonicity, integral nonlinearity  (INL) of ±16 LSB maximum, low noise, and 10 μs typical  settling time.

The AD5722/AD5732/AD5752 use a serial interface that  operates at clock rates up to 30 MHz and are compatible with  DSP and microcontroller interface standards. Double buffering  allows the simultaneous updating of all DACs. The input coding  is user-selectable twos complement or offset binary for a bipolar  output (depending on the state of Pin BIN/2sComp), and  straight binary for a unipolar output. The asynchronous clear  function clears all DAC registers to a user-selectable zero-scale  or midscale output. The parts are available in a 24-lead TSSOP  and offer guaranteed specifications over the −40°C to +85°C  industrial temperature range.

The AD5722/AD5732/AD5752 are pin compatible with the  AD5724/AD5734/AD5754, which are complete, quad, 12-/14-/  16-bit, serial input, unipolar/bipolar voltage output DACs.

DVCC

FUNCTIONAL BLOCK DIAGRAM

AVSS AVDD

AD5722/AD5732/AD5752

REFIN

CLR

BIN/2sCOMP SDIN

SCLK

SYNC

SDO

INPUT SHIFT REGISTER AND

CONTROL LOGIC

12/14/16

INPUT

REGISTER A

INPUT

REGISTER B

DAC

REGISTER A

DAC

REGISTER B

12/14/16 12/14/16

REFERENCE BUFFERS

DAC A

DAC B

VOUTA VOUTB

06467-001

GND DAC_GND (2) SIG_GND (2) LDAC

Figure 1.

1 For analog systems designers within industrial/instrumentation equipment OEMs who need high performance ICs at higher voltage levels, iCMOS® is a technology  platform that enables the development of analog ICs capable of 30 V and operating at ±15 V supplies while allowing dramatic reductions in power consumption and  package size, as well as increased ac and dc performance.

Rev. F Document Feedback

Information furnished by Analog Devices is believed to be accurate and reliable. However, no

responsibility is assumed by Analog Devices for its use, nor for any infringements of patents or other  rights of third parties that may result from its use. Specifications subject to change without notice. No  license is granted by implication or otherwise under any patent or patent rights of Analog Devices.  Trademarks and registered trademarks are the property of their respective owners.

One Technology Way, P.O. Box 9106, Norwood, MA 02062-9106, U.S.A. Tel: 781.329.4700 ©2008–2017 Analog Devices, Inc. All rights reserved.  Technical Support www.analog.com

AD5722/AD5732/AD5752 Data Sheet

TABLE OF CONTENTS

Features .............................................................................................. 1 Applications ....................................................................................... 1 General Description ......................................................................... 1 Functional Block Diagram .............................................................. 1 Revision History ............................................................................... 2 Specifications ..................................................................................... 3

AC Performance Characteristics ................................................ 5 Timing Characteristics ................................................................ 5 Timing Diagrams .......................................................................... 6

Absolute Maximum Ratings ............................................................ 8 ESD Caution .................................................................................. 8 Pin Configuration and Function Descriptions ............................. 9 Typical Performance Characteristics ........................................... 10 Terminology .................................................................................... 16 Theory of Operation ...................................................................... 18 Architecture ................................................................................. 18 Power-Up Sequence ................................................................... 18 Serial Interface ............................................................................ 18 Load DAC (LDAC) ..................................................................... 20 Asynchronous Clear (CLR) ....................................................... 20 Configuring the AD5722/AD5732/AD5752 .......................... 20

REVISION HISTORY

2/2017—Rev. E to Rev. F

Added Power-Up Sequence Section ............................................. 18  Changes to Table 7 and Table 8 ..................................................... 21  Changes to Table 10 and Table 11 ................................................ 22  Changes to Table 13 and Table 14 ................................................ 23  Changes to Analog Output Control Section ............................... 27  Added Alternative Power-Up Sequence Support Section,

Figure 43, and Figure 44; Renumbered Sequentially ................. 28

2/2016—Rev. D to Rev. E

Changes to Table 1 ............................................................................ 3

7/2011—Rev. C to Rev. D

Changes to Table 3: t7, t8, t10 Limits ...................................................... 5

Transfer Function ....................................................................... 20 Input Shift Register .................................................................... 24 DAC Register .............................................................................. 25 Output Range Select Register ................................................... 25 Control Register ......................................................................... 26 Power Control Register ............................................................. 26

Design Features ............................................................................... 27 Analog Output Control ............................................................. 27 Power-Down Mode .................................................................... 27 Overcurrent Protection ............................................................. 27 Thermal Shutdown .................................................................... 27

Applications Information .............................................................. 28 +5 V/±5 V Operation ................................................................ 28 Alternative Power-Up Sequence Support ............................... 28 Layout Guidelines....................................................................... 28 Galvanically Isolated Interface ................................................. 29 Voltage Reference Selection ...................................................... 29 Microprocessor Interfacing ....................................................... 29

Outline Dimensions ....................................................................... 31 Ordering Guide .......................................................................... 31

3/2011—Rev. B to Rev. C

Changes to Configuring the AD5722/AD5732/AD5752 Section .. 20

8/2010—Rev. A to Rev. B

Changes to Table 27 ....................................................................... 26

5/2010—Rev. 0 to Rev. A

Changes to Junction Temperature, TJ max Parameter, Table 4 ... 8  Changes to Exposed Paddle Description, Table 5 ......................... 9  Changes to Ordering Guide .......................................................... 30

10/08—Revision 0: Initial Version

Rev. F | Page 2 of 31

Data Sheet AD5722/AD5732/AD5752

SPECIFICATIONS

AVDD = 4.5 V1 to 16.5 V; AVSS = −4.5 V1 to −16.5 V, or AVSS = 0 V; GND = 0 V; REFIN = 2.5 V; DVCC = 2.7 V to 5.5 V; RLOAD = 2 kΩ;  CLOAD = 200 pF; all specifications TMIN to TMAX, unless otherwise noted.

Table 1.

| Min Typ Max | Unit |
| --- | --- |
| 16 14 12 −0.1 +0.1 −0.3 +0.3 −16 +16 −4 +4 −1 +1 −1 +1 −6 +6 ±4 −6 +6 ±4 −6 +6 ±4 −0.025 +0.025 −0.065 0 0 0.08 ±4 120 | Bits Bits Bits % FSR % FSR LSB LSB LSB LSB mV ppm FSR/°C mV ppm FSR/°C mV ppm FSR/°C % FSR ppm FSR/°C μV |
| 2.5 1 5 −2 ±0.5 +2 2 3 | V MΩ μA V |
| −10.8 +10.8 −12 +12 0.5 0.9 ±4 ±50 20 2 4000 0.5 | V V V ppm FSR/°C ppm FSR mA kΩ pF Ω |

Parameter Test Conditions/Comments  ACCURACY Outputs unloaded

Resolution

AD5752

AD5732

AD5722

Total Unadjusted Error (TUE)

B Version ±10 V range

A Version ±10 V range

Integral Nonlinearity (INL)2

AD5752 A, B Versions

AD5732 A Version

AD5722 A Version

Differential Nonlinearity (DNL) All models, all versions, guaranteed monotonic  Bipolar Zero Error ±10 V range, TA = 25°C, error at other temperatures  obtained using bipolar zero TC

Bipolar Zero TC3

Zero-Scale Error ±10 V range, TA = 25°C, error at other temperatures  obtained using zero-scale TC

Zero-Scale TC3

Offset Error +10 V range, TA = 25°C, error at other temperatures  obtained using zero-scale TC

Offset Error TC

Gain Error ±10 V range, TA = 25°C, error at other temperatures  obtained using gain TC

Gain Error3 +10 V and +5 V ranges, TA = 25°C, error at other  temperatures obtained using gain TC

Gain Error3 ±5 V range, TA = 25°C, error at other temperatures  obtained using gain TC

Gain TC3

DC Crosstalk3

REFERENCE INPUT3

Reference Input Voltage ±1% for specified performance  DC Input Impedance

Input Current

Reference Range

OUTPUT CHARACTERISTICS3

Output Voltage Range AVDD/AVSS = ±11.7 V min, REFIN = +2.5 V   AVDD/AVSS = ±12.9 V min, REFIN = +3 V  Headroom Required

Output Voltage TC

Output Voltage Drift vs. Time Drift after 1000 hours of life test @ 125°C  Short-Circuit Current

Load For specified performance  Capacitive Load Stability

DC Output Impedance

Rev. F | Page 3 of 31

AD5722/AD5732/AD5752 Data Sheet

| Min Typ Max | Unit |
| --- | --- |
| 2 0.8 ±1 5 | V V μA pF |
| 0.4 DVCC − 1 0.4 DVCC − 0.5 −1 +1 5 | V V V V μA pF |
| 4.5 16.5 −4.5 −16.5 2.7 5.5 −65 3.25 2.4 2.5 0.5 3 190 79 40 40 300 | V V V dB mA/channel mA/channel mA/channel μA mW mW μA μA nA |

Parameter Test Conditions/Comments  DIGITAL INPUTS3 DVCC = 2.7 V to 5.5 V, JEDEC compliant  Input High Voltage, VIH

Input Low Voltage, VIL

Input Current Per pin

Pin Capacitance Per pin

DIGITAL OUTPUTS (SDO)3

Output Low Voltage, VOL DVCC = 5 V ± 10%, sinking 200 μA  Output High Voltage, VOH DVCC = 5 V ± 10%, sourcing 200 μA  Output Low Voltage, VOL DVCC = 2.7 V to 3.6 V, sinking 200 μA  Output High Voltage, VOH DVCC = 2.7 V to 3.6 V, sourcing 200 μA  High Impedance Leakage

Current

High Impedance Output

Capacitance

POWER REQUIREMENTS

AVDD

AVSS

DVCC

Power Supply Sensitivity3

∆VOUT/∆ΑVDD

AIDD Outputs unloaded

AVSS = 0 V, outputs unloaded  AISS Outputs unloaded

DICC VIH = DVCC, VIL = GND

Power Dissipation ±16.5 V operation, outputs unloaded  16.5 V operation, AVSS = 0 V, outputs unloaded

Power-Down Currents

AIDD

AISS

DICC

1 For specified performance, the maximum headroom requirement is 0.9 V.

2 INL is the relative accuracy. It is measured from Code 512, Code 128, and Code 32 for the AD5752, the AD5732, and the AD5722, respectively. 3 Guaranteed by characterization; not production tested.

Rev. F | Page 4 of 31

Data Sheet AD5722/AD5732/AD5752

AC PERFORMANCE CHARACTERISTICS

AVDD = 4.5 V1 to 16.5 V; AVSS = −4.5 V to −16.5 V, or AVSS = 0 V; GND = 0 V; REFIN = 2.5 V; DVCC = 2.7 V to 5.5 V; RLOAD = 2 kΩ;  CLOAD = 200 pF; all specifications TMIN to TMAX, unless otherwise noted.

Table 2.

| Min Typ Max | Unit |
| --- | --- |
| 10 12 7.5 8.5 5 3.5 13 35 10 10 0.6 15 80 320 | μs μs μs V/μs nV-sec mV nV-sec nV-sec nV-sec μV p-p μV rms nV/√Hz |

Parameter2 Test Conditions/Comments  DYNAMIC PERFORMANCE

Output Voltage Settling Time 20 V step to ±0.03% FSR  10 V step to ±0.03% FSR

512 LSB step settling (16-bit resolution)

Slew Rate

Digital-to-Analog Glitch Energy

Glitch Impulse Peak Amplitude

Digital Crosstalk

DAC-to-DAC Crosstalk

Digital Feedthrough

Output Noise

0.1 Hz to 10 Hz Bandwidth 0x8000 DAC code

100 kHz Bandwidth

Output Noise Spectral Density Measured at 10 kHz, 0x8000 DAC code

1 For specified performance, the maximum headroom requirement is 0.9 V.

2 Guaranteed by design and characterization; not production tested.

TIMING CHARACTERISTICS

AVDD = 4.5 V to 16.5 V; AVSS = −4.5 V to −16.5 V, or AVSS = 0 V; GND = 0 V; REFIN = 2.5 V; DVCC = 2.7 V to 5.5 V; RLOAD = 2 kΩ; CLOAD =  200 pF; all specifications tMIN to tMAX, unless otherwise noted.

Table 3.

| Limit at tMIN, tMAX | Unit |
| --- | --- |
| 33 13 13 13 13 100 7 2 20 130 20 10 20 2.5 13 40 200 | ns min ns min ns min ns min ns min ns min ns min ns min ns min ns min ns min μs max ns min μs max ns min ns max ns min |

Parameter1, 2, 3 Description

t1 SCLK cycle time

t2 SCLK high time

t3 SCLK low time

t4 SYNC falling edge to SCLK falling edge setup time  t5 SCLK falling edge to SYNC rising edge

t6 Minimum SYNC high time (write mode)

t7 Data setup time

t8 Data hold time

t9 LDAC falling edge to SYNC falling edge

t10 SYNC rising edge to LDAC falling edge

t11 LDAC pulse width low

t12 DAC output settling time

t13 CLR pulse width low

t14 CLR pulse activation time

t154 SYNC rising edge to SCLK falling edge

t164 SCLK rising edge to SDO valid (CL SDO5 = 15 pF)  t17 Minimum SYNC high time (readback/daisy-chain mode)

1 Guaranteed by characterization; not production tested.

2 All input signals are specified with tR = tF = 5 ns (10% to 90% of DVCC) and timed from a voltage level of 1.2 V.

3 See Figure 2, Figure 3, and Figure 4.

4 Daisy-chain and readback mode.

5 CL SDO = capacitive load on SDO output.

Rev. F | Page 5 of 31

AD5722/AD5732/AD5752 Data Sheet

TIMING DIAGRAMS

t1

SCLK SYNC

21 42

t6 t3 t2 t5

t4

SDIN LDAC VOUTx

VOUTx CLR

VOUTx

t9

t7

DB23

t13

t14

t8

DB0

t10t11 t12

t12

06467-002

Figure 2. Serial Interface Timing Diagram

t1

SCLK 42 84

t17

SYNC

t4

t7

t3 t2 t5 t15

t8

D32B D0B D32B D0B SDIN

INPUT WORD FOR DAC N INPUT WORD FOR DAC N – 1

t16

SDO LDAC

DB23

UNDEFINED INPUT WORD FOR DAC N Figure 3. Daisy-Chain Timing Diagram

DB0

t10

t11

06467-003

Rev. F | Page 6 of 31

Data Sheet AD5722/AD5732/AD5752

SCLK 24 24

1 1

t17

SYNC

DB23 DB0 DB23 DB0 SDIN

INPUT WORD SPECIFIES NOP CONDITION

REGISTER TO BE READ

DB23 DB0 DB23 DB0 SDO

UNDEFINED

06467-004

Figure 4. Readback Timing Diagram

SELECTED REGISTER DATA CLOCKED OUT

Rev. F | Page 7 of 31

AD5722/AD5732/AD5752 Data Sheet

ABSOLUTE MAXIMUM RATINGS

TA = 25°C unless otherwise noted.

Transient currents of up to 100 mA do not cause SCR latch-up.

Table 4.

Parameter Rating

AVDD to GND −0.3 V to +17 V  AVSS to GND +0.3 V to −17 V  DVCC to GND −0.3 V to +7 V  Digital Inputs to GND −0.3 V to DVCC + 0.3 V or 7 V  (whichever is less)

Digital Outputs to GND −0.3 V to DVCC + 0.3 V or 7 V  (whichever is less)

REFIN to GND −0.3 V to +5 V  VOUTA or VOUTB to GND AVSS to AVDD DAC_GND to GND −0.3 V to +0.3 V  SIG_GND to GND −0.3 V to +0.3 V  Operating Temperature Range, TA

Industrial −40°C to +85°C  Storage Temperature Range −65°C to +150°C  Junction Temperature, TJ max 150°C

24-Lead TSSOP Package

θJA Thermal Impedance 42°C/W

θJC Thermal Impedance 9°C/W

Power Dissipation (TJ max − TA)/θJA Lead Temperature JEDEC industry standard  Soldering J-STD-020

ESD (Human Body Model) 3.5 kV

Stresses at or above those listed under Absolute Maximum  Ratings may cause permanent damage to the product. This is a  stress rating only; functional operation of the product at these  or any other conditions above those indicated in the operational  section of this specification is not implied. Operation beyond  the maximum operating conditions for extended periods may  affect product reliability.

ESD CAUTION

Rev. F | Page 8 of 31

Data Sheet AD5722/AD5732/AD5752

PIN CONFIGURATION AND FUNCTION DESCRIPTIONS

AVSS

NC

VOUTA

NC

BIN/2sCOMP NC

SYNC

SCLK

SDIN

LDAC

CLR

NC

NOTES

1

2

3

4

5

6

7

8

9

10 11 12

AD5722/ AD5732/ AD5752

TOP VIEW (Not to Scale)

24 23 22 21 20 19 18 17 16 15 14 13

AVDD

VOUTB

NC

SIG_GND SIG_GND DAC_GND DAC_GND REFIN

SDO

GND

DVCC

NC

1. NC = NO CONNECT

2. IT IS RECOMMENDED THAT THE

EXPOSED PAD BE THERMALLY

06467-005

CONNECTED TO A COPPER PLANE

FOR ENHANCED THERMAL

PERFORMANCE.

Figure 5. Pin Configuration

Table 5. Pin Function Descriptions

Mnemonic

AVSS NC VOUTA BIN/2sCOMP SYNC SCLK SDIN LDAC CLR DVCC GND SDO REFIN DAC_GND SIG_GND VOUTB AVDD

Pin No. Description

1 Negative Analog Supply. Voltage ranges from −4.5 V to −16.5 V. This pin can be connected to 0 V if output  ranges are unipolar.

2, 4, 6, 12,  13, 22

Do not connect to these pins.

3 Analog Output Voltage of DAC A. The output amplifier is capable of directly driving a 2 kΩ, 4000 pF load.  5 Determines the DAC coding for a bipolar output range. This pin should be hardwired to either DVCC or GND.  When hardwired to DVCC, input coding is offset binary. When hardwired to GND, input coding is twos

complement. (For unipolar output ranges, coding is always straight binary.)

7 Active Low Input. This is the frame synchronization signal for the serial interface. While SYNC is low, data is  transferred on the falling edge of SCLK. Data is latched on the rising edge of SYNC.

8 Serial Clock Input. Data is clocked into the shift register on the falling edge of SCLK. This operates at clock  speeds up to 30 MHz.

9 Serial Data Input. Data must be valid on the falling edge of SCLK.

10 Load DAC, Logic Input. This is used to update the DAC registers and, consequently, the analog outputs. When  this pin is tied permanently low, the addressed DAC register is updated on the rising edge of SYNC. If LDAC is  held high during the write cycle, the DAC input register is updated, but the output update is held off until the  falling edge of LDAC. In this mode, all analog outputs can be updated simultaneously on the falling edge of  LDAC. The LDAC pin should not be left unconnected.

11 Active Low Input. Asserting this pin sets the DAC registers to zero-scale code or midscale code (user-selectable).  14 Digital Supply. Voltage ranges from 2.7 V to 5.5 V.

15 Ground Reference.

16 Serial Data Output. Used to clock data from the serial register in daisy-chain or readback mode. Data is clocked  out on the rising edge of SCLK and is valid on the falling edge of SCLK.

17 External Reference Voltage Input. Reference input range is 2 V to 3 V. REFIN = 2.5 V for specified performance.  18, 19 Ground Reference for the Two Digital-to-Analog Converters (DACs).

20, 21 Ground Reference for the Two Output Amplifiers.

23 Analog Output Voltage of DAC B. The output amplifier is capable of directly driving a 2 kΩ, 4000 pF load.  24 Positive Analog Supply. Voltage ranges from 4.5 V to 16.5 V.

Exposed  Paddle

This exposed paddle should be connected to the potential of the AVSS pin, or alternatively, it can be left electrically  unconnected. It is recommended that the paddle be thermally connected to a copper plane for enhanced thermal  performance.

Rev. F | Page 9 of 31

AD5722/AD5732/AD5752 Data Sheet

TYPICAL PERFORMANCE CHARACTERISTICS

6 AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V AV/AV = ±6.5V, RANGE = ±5V

0.6

INL ERROR (LSB)

4

0.4

|  | DDAVDD | SS/AVSS | = +6.5V/0V, RANGE = +5V |  |
| --- | --- | --- | --- | --- |

2

0.2

DNL ERROR (LSB)

0

0

–2

–0.2

–4

–0.4

–6

–0.6

| /AVSS |  | = +12V/0V, RANGE = +10V |  |  |
| --- | --- | --- | --- | --- |

AVDDAVDD/AVSS = ±12V, RANGE = ±10V

AVDD/AVSS = ±6.5V, RANGE = ±5V

0 10,000 20,000 30,000 40,000 50,000 60,000 –8 06467-013

CODE

Figure 6. AD5752 Integral Nonlinearity Error vs. Code

AVDD/AVSS = +6.5V/0V, RANGE = +5V

0 10,000 20,000 30,000 40,000 50,000 60,000 –0.8 06467-016

CODE

Figure 9. AD5752 Differential Nonlinearity Error vs. Code

INL ERROR (LSB)

1.5

1.0

0.5

0

–0.5 –1.0 –1.5 –2.0

AVDD/AVSS = +12V/0V, RANGE = +10V

AVDD/AVSS = ±12V, RANGE = ±10V AVDD/AVSS = ±6.5V, RANGE = ±5V

|  |  | AVDD | /AVSS |  | = +6.5V/0V, RANGE = +5V |  |
| --- | --- | --- | --- | --- | --- | --- |

DNL ERROR (LSB)

06467-014

0 2000 4000 6000 8000 10,000 12,000 14,000 16,000

CODE

0.15 0.10 0.05 0

–0.05 –0.10 –0.15 –0.20

| DD/AVSS |  |  | = +12V/0V, RANGE = +10V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

AVAVDD/AVSS = ±12V, RANGE = ±10V AVDD/AVSS = ±6.5V, RANGE = ±5V

AVDD/AVSS = +6.5V/0V, RANGE = +5V

06467-017

0 2000 4000 6000 8000 10,000 12,000 14,000 16,000 CODE

Figure 7. AD5732 Integral Nonlinearity Error vs. Code

Figure 10. AD5732 Differential Nonlinearity Error vs. Code

0.3

AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V

0.04 0.03

AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V

INL ERROR (LSB)

0.2

0.1

0

–0.1 –0.2 –0.3 –0.4

AV/AV = ±6.5V, RANGE = ±5V

|  |  | DDAVDD | SS/AVSS | = +6.5V/0V, RANGE = +5V |  |  |
| --- | --- | --- | --- | --- | --- | --- |

DNL ERROR (LSB)

AVAV

0.02

0.01

0

–0.01

–0.02

–0.03

–0.04

| DD/AVSSDD/AVSS |  | = ±6.5V, RANGE = ±5V= +6.5V/0V, RANGE = +5V |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

–0.5

06467-015

0 500 1000 1500 2000 2500 3000 3500 4000 CODE

–0.05

06467-018

0 500 1000 1500 2000 2500 3000 3500 4000 CODE

Figure 8. AD5722 Integral Nonlinearity Error vs. Code

Figure 11. AD5722 Differential Nonlinearity Error vs. Code

Rev. F | Page 10 of 31

Data Sheet AD5722/AD5732/AD5752

8

6

4

INL ERROR (LSB)

10

8

|  |  |  |  | MAX INL ±10V MAX INL ±5V |
| --- | --- | --- | --- | --- |
|  |  |  |  | MIN INL ±10V MIN INL ±5V MAX INL +10V |
|  |  |  |  | MIN INL +10V MAX INL +5V MIN INL +5V |

6

4

|  |  |  |  |  |  |  | BIPOLAR 5V MINUNIPOLAR 5V MIN |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BIPOLAR 5V MAXUNIPOLAR 5V MAX |  |

2

0

–2

–4

–6

–8

06467-044

–40 –20 0 20 40 60 80 TEMPERATURE (°C)

Figure 12. AD5752 Integral Nonlinearity Error vs. Temperature

INL ERROR (LSB)

2

0

–2

–4

–6

–8

5.5 8.5 6.5 7.5 10.5 11.5 9.5 12.5 13.5 14.5 15.5 16.5 –10 06467-035

SUPPLY VOLTAGE (V)

Figure 15. AD5752 Integral Nonlinearity Error vs. Supply Voltage

0.1

1.0 0.8

BIPOLAR 10V MIN UNIPOLAR 10V MIN

DNL ERROR (LSB)

0

–0.1 –0.2 –0.3 –0.4 –0.5 –0.6

|  |  |  |  | MAX DNL ±10VMAX DNL ±5V MIN DNL ±10V MIN DNL ±5V |
| --- | --- | --- | --- | --- |
|  |  |  |  | MAX DNL +10VMIN DNL +10V MAX DNL +5V |
|  |  |  |  | MIN DNL +5V |

0.6

0.4

DNL ERROR (LSB)

0.2

0

–0.2

–0.4

–0.6

–0.8

–1.0

|  |  |  |  |  |  |  | BIPOLAR 10V MAXUNIPOLAR 10V MAX |
| --- | --- | --- | --- | --- | --- | --- | --- |

06467-032

06467-045

–40 –20 0 20 40 60 80 TEMPERATURE (°C)

Figure 13. AD5752 Differential Nonlinearity Error vs. Temperature

11.5 12.512.0 13.0 13.5 14.0 14.5 15.0 15.5 16.0 16.5 SUPPLY VOLTAGE (V)

Figure 16. AD5752 Differential Nonlinearity Error vs. Supply Voltage

10

1.0 0.8

BIPOLAR 5V MIN UNIPOLAR 5V MIN

INL ERROR (LSB)

8

6

4

2

0

–2

–4

–6

–8

–10

|  |  |  |  |  |  | BIPOLAR 10V MINUNIPOLAR 10V MIN |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BIPOLAR 10V MAXUNIPOLAR 10V MAX |  |

0.6

0.4

DNL ERROR (LSB)

0.2

0

–0.2

–0.4

–0.6

–0.8

–1.0

|  |  |  |  |  |  |  |  | BIPOLAR 5V MAXUNIPOLAR 5V MAX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

06467-033

06467-034

11.5 12.512.0 13.0 13.5 14.0 14.5 15.0 15.5 16.0 16.5 SUPPLY VOLTAGE (V)

Figure 14. AD5752 Integral Nonlinearity Error vs. Supply Voltage

5.5 6.5 7.5 8.5 9.5 10.5 11.5 12.5 13.5 14.5 15.5 16.5 SUPPLY VOLTAGE (V)

Figure 17. AD5752 Differential Nonlinearity Error vs. Supply Voltage

Rev. F | Page 11 of 31

AD5722/AD5732/AD5752 Data Sheet

TUE (%)

0.02

0.01

0

–0.01 –0.02 –0.03 –0.04

6.0

5.5

|  |  |  |  |  |  | BIPOLAR 10V MIN |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | UNIPOLAR 10V MINBIPOLAR 10V MAXUNIPOLAR 10V MAX |  |

5.0

4.5

AIDD (mA)

4.0

3.5

3.0

2.5

2.0

06467-036

06467-042

11.5 12.5 12.0 13.5 14.0 13.0 14.5 15.0 15.5 16.0 16.5 SUPPLY VOLTAGE (V)

Figure 18. AD5752 Total Unadjusted Error vs. Supply Voltage  0.04

4.5 6.5 8.5 10.5 12.5 14.5 16.5 AVDD (V)

Figure 21. Supply Current vs. Supply Voltage (Single Supply)  4

TUE (%)

0.03 0.02 0.01

0

–0.01 –0.02 –0.03 –0.04

|  |  |  |  |  |  |  |  | BIPOLAR 5V MIN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | UNIPOLAR 5V MINBIPOLAR 5V MAX |
|  |  |  |  |  |  |  |  | UNIPOLAR 5V MAX |

3

ZERO-SCALE ERROR (mV)

2

1

0

–1

–2

+10V

|  | ±10V |  |  |  |
| --- | --- | --- | --- | --- |

±5V

5.5 8.5 6.5 7.5 10.5 11.5 9.5 12.5 13.5 14.5 15.5 16.5 –0.05 06467-037

SUPPLY VOLTAGE (V)

Figure 19. AD5752 Total Unadjusted Error vs. Supply Voltage  5

–3

06467-046

–40 –20 0 20 40 60 80 TEMPERATURE (°C)

Figure 22. Zero-Scale Error vs. Temperature

0.8

4

3

2

AIDD/AISS (mA)

1

0

–1

–2

–3

–4

06467-038

0.6

|  | AIDD | (mA) |  |
| --- | --- | --- | --- |
|  | AI | (mA) |  |
|  | DD |  |  |

BIPOLAR ZERO ERROR (mV)

0.4

0.2

0

–0.2

–0.4

–0.6

–0.8

–1.0

|  | ±5V RANGE |  |  |  |
| --- | --- | --- | --- | --- |
| ±10V RANGE |  |  |  |  |

4.5 6.5 8.5 10.5 12.5 14.5 16.5 AVDD/AVSS (V)

Figure 20. Supply Current vs. Supply Voltage (Dual Supply)

06467-047

–40 –20 0 20 40 60 80 TEMPERATURE (°C)

Figure 23. Bipolar Zero Error vs. Temperature

Rev. F | Page 12 of 31

Data Sheet AD5722/AD5732/AD5752

0.06

±5V

15

GAIN ERROR (% FSR)

0.04 0.02 0

10

|  | ±10V |  |  |  |
| --- | --- | --- | --- | --- |
|  | +10V |  |  |  |

OUTPUT VOLTAGE (V)

5

0

–0.02

–5

–0.04

–10

–0.06

–15

06467-048

–40 –20 0 20 40 60 80

TEMPERATURE (°C)

Figure 24. Gain Error vs. Temperature

1000

7

06467-022

–3 –1 1 3 5 7 9 11 TIME (µs)

Figure 27. Full-Scale Settling Time, ±10 V Range

DICC (µA)

900

800

700

600

500

400

300

200

100

0

–100

|  |  | DVCC | = 5V |
| --- | --- | --- | --- |
|  | DVCC = 3V |  |  |

5

OUTPUT VOLTAGE (V)

3

1

–1

–3

–5

–7

06467-043

0123456

VLOGIC (V)

Figure 25. Digital Current vs. Logic Input Voltage

06467-023

–3 –1 1 3 5 7 9 11 TIME (µs)

Figure 28. Full-Scale Settling Time, ±5 V Range

OUTPUT VOLTAGE DELTA (V)

0.010 0.005 0

–0.005 –0.010 –0.015 –0.020

12

±5V RANGE, CODE = 0xFFFF

±10V RANGE, CODE = 0xFFFF

+10V RANGE, CODE = 0xFFFF

+5V RANGE, CODE = 0xFFFF

| ±5V RANGE, CODE = 0x0000±10V RANGE, CODE = 0x0000 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

10

OUTPUT VOLTAGE (V)

8

6

4

2

06467-040

0

–25 –20 –15 –10 –5 0 5 10 15 20 25 OUTPUT CURRENT (mA)

Figure 26. Output Source and Sink Capability

06467-024

–3 –1 1 3 5 7 9 11 TIME (µs)

Figure 29. Full-Scale Settling Time, +10 V Range

Rev. F | Page 13 of 31

AD5722/AD5732/AD5752 Data Sheet

6

OUTPUT VOLTAGE (V)

5

4

3

1

2

1

0

06467-025

–3 –1 1 3 5 7 9 11

RANGE = ±5V RANGE = +5V

RANGE = +10V RANGE = ±10V

06467-027

TIME (µs)

Figure 30. Full-Scale Settling Time, +5 V Range

CH1 5µV M5s LINE 73.8V Figure 33. Peak-to-Peak Noise, 100 kHz Bandwidth

0.020 0.015

±10V RANGE, 0x7FFF TO 0x8000 ± ±10V RANGE, 0x8000 TO 0x7FFF ±5V RANGE, 0x7FFF TO 0x8000

0.10 0.08

AVDD/AVSS = ±16.5V

AVDD = +16.5V, AVSS = 0V

OUTPUT VOLTAGE (V)

0.010 0.005 0

–0.005 –0.010 –0.015

|  |  | ±5V RANGE, 0x8000 TO 0x7FFF+10V RANGE, 0x7FFF TO 0x8000+10V RANGE, 0x8000 TO 0x7FFF+5V RANGE, 0x7FFF TO 0x8000 |  |
| --- | --- | --- | --- |
|  |  | +5V RANGE, 0x8000 TO 0x7FFF |  |

06467-039

OUTPUT VOLTAGE (V)

0.06 0.04 0.02

0

–0.02 –0.04 –0.06

06467-041

–1 0 1 2 3 4 5

TIME (µs)

Figure 31. Digital-to-Analog Glitch Energy

15

10

5

0

TUE (LSB)

–5

–10

1

–15

–20

–25

–30

–50 –30 –10 10 30 50 70 90 TIME (µs)

Figure 34. Output Glitch on Power-Up

AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V

AVDDAVDD

| /AVSS /AVSS | = ±6.5V, RANGE = ±5V = +6.5V/0V, RANGE = +5V |  |  |  |
| --- | --- | --- | --- | --- |

RANGE = ±5V RANGE = +5V

RANGE = +10V RANGE = ±10V

06467-026

–35

06467-019

0 1000 2000 3000 4000 5000 6000

CH1 5µV M 5s LINE 73.8V Figure 32. Peak-to-Peak Noise, 0.1 Hz to 10 Hz Bandwidth

CODE

Figure 35. AD5752 Total Unadjusted Error vs. Code

Rev. F | Page 14 of 31

Data Sheet AD5722/AD5732/AD5752

4

AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V AVDD/AVSS = ±6.5V, RANGE = ±5V 2

1.0 0.5

AVDD/AVSS = +12V/0V, RANGE = +10V AVDD/AVSS = ±12V, RANGE = ±10V AVDD/AVSS = ±6.5V, RANGE = ±5V

TUE (LSB)

AV

0

–2

–4

–6

–8

AV

| DD/AVSS |  |  | = +6.5V/0V, RANGE = +5V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0

TUE (LSB)

–0.5

–1.0

–1.5

–2.0

| DD/AVSS |  |  | = +6.5V/0V, RANGE = +5V |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

–10

06467-020

0 2000 4000 6000 8000 10000 12000 14000 16000 CODE

Figure 36. AD5732 Total Unadjusted Error vs. Code

–2.5

06467-021

0 500 1000 1500 2000 2500 3000 3500 4000 CODE

Figure 37. AD5722 Total Unadjusted Error vs. Code

Rev. F | Page 15 of 31

AD5722/AD5732/AD5752 Data Sheet

TERMINOLOGY

Relative Accuracy or Integral Nonlinearity (INL)  For the DAC, relative accuracy, or integral nonlinearity, is a  measure of the maximum deviation in LSBs from a straight line  passing through the endpoints of the DAC transfer function. A  typical INL vs. code plot can be seen in Figure 6.

Differential Nonlinearity (DNL)

Differential nonlinearity is the difference between the measured  change and the ideal 1 LSB change between any two adjacent  codes. A specified differential nonlinearity of ±1 LSB maximum  ensures monotonicity. This DAC is guaranteed monotonic by  design. A typical DNL vs. code plot can be seen in Figure 9.

Monotonicity

A DAC is monotonic if the output either increases or remains  constant for increasing digital input code. The AD5722/AD5732/  AD5752 are monotonic over their full operating temperature  range.

Bipolar Zero Error

Bipolar zero error is the deviation of the analog output from the  ideal half-scale output of 0 V when the DAC register is loaded  with 0x8000 (straight binary coding) or 0x0000 (twos complement  coding). A plot of bipolar zero error vs. temperature can be seen  in Figure 23.

Bipolar Zero TC

Bipolar zero TC is a measure of the change in the bipolar zero  error with a change in temperature. It is expressed in ppm FSR/°C.

Zero-Scale Error or Negative Full-Scale Error

Zero-scale error is the error in the DAC output voltage when  0x0000 (straight binary coding) or 0x8000 (twos complement  coding) is loaded to the DAC register. Ideally, the output voltage  should be negative full-scale − 1 LSB. A plot of zero-scale error  vs. temperature can be seen in Figure 22.

Zero-Scale TC

Zero-scale TC is a measure of the change in zero-scale error with a  change in temperature. Zero-scale TC is expressed in ppm FSR/°C.

Output Voltage Settling Time

Output voltage settling time is the amount of time required for  the output to settle to a specified level for a full-scale input change.  A plot for full-scale settling time can be seen in Figure 27.

Slew Rate

The slew rate of a device is a limitation in the rate of change of  the output voltage. The output slewing speed of a voltage output  DAC is usually limited by the slew rate of the amplifier used at  its output. Slew rate is measured from 10% to 90% of the output  signal and is given in V/μs.

Gain Error

Gain error is a measure of the span error of the DAC. It is the  deviation in slope of the DAC transfer characteristic from the  ideal and is expressed in % FSR. A plot of gain error vs.  temperature can be seen in Figure 24.

Gain TC

Gain TC is a measure of the change in gain error with changes  in temperature. Gain TC is expressed in ppm FSR/°C.

Total Unadjusted Error (TUE)

Total unadjusted error is a measure of the output error taking  all the various errors into account, namely INL error, offset  error, gain error, and output drift over supplies, temperature,  and time. TUE is expressed in % FSR.

Digital-to-Analog Glitch Impulse

Digital-to-analog glitch impulse is the impulse injected into the  analog output when the input code in the DAC register changes  state, but the output voltage remains constant. It is normally  specified as the area of the glitch in nV-sec and is measured  when the digital input code is changed by 1 LSB at the major  carry transition (0x7FFF to 0x8000). See Figure 31.

Glitch Impulse Peak Amplitude

Glitch impulse peak amplitude is the peak amplitude of the  impulse injected into the analog output when the input code in  the DAC register changes state. It is specified as the amplitude  of the glitch in mV and is measured when the digital input code  is changed by 1 LSB at the major carry transition (0x7FFF to  0x8000). See Figure 31.

Digital Feedthrough

Digital feedthrough is a measure of the impulse injected into  the analog output of the DAC from the digital inputs of the  DAC but is measured when the DAC output is not updated. It is  specified in nV-sec and measured with a full-scale code change  on the data bus.

Power Supply Sensitivity

Power supply sensitivity indicates how the output of the DAC is  affected by changes in the power supply voltage. It is measured  by superimposing a 50 Hz/60 Hz, 200 mV p-p sine wave on the  supply voltages and measuring the proportion of the sine wave  that transfers to the outputs.

DC Crosstalk

This is the dc change in the output level of one DAC in response  to a change in the output of another DAC. It is measured with a  full-scale output change on one DAC while monitoring another  DAC. It is expressed in LSBs.

Rev. F | Page 16 of 31

Data Sheet AD5722/AD5732/AD5752

Digital Crosstalk

Digital crosstalk is a measure of the impulse injected into the  analog output of one DAC from the digital inputs of another  DAC but is measured when the DAC output is not updated. It is  specified in nV-sec and measured with a full-scale code change  on the data bus.

DAC-to-DAC Crosstalk

DAC-to-DAC crosstalk is the glitch impulse transferred to the  output of one DAC due to a digital code change and a subsequent  output change of another DAC. This includes both digital and

analog crosstalk. It is measured by loading one of the DACs  with a full-scale code change (all 0s to all 1s and vice versa) with  LDAC low and monitoring the output of another DAC. The  energy of the glitch is expressed in nV-sec.

Rev. F | Page 17 of 31

AD5722/AD5732/AD5752 Data Sheet

THEORY OF OPERATION

The AD5722/AD5732/AD5752 are dual, 12-/14-/16-bit, serial  input, unipolar/bipolar, voltage output DACs. They operate  from unipolar supply voltages of +4.5 V to +16.5 V or bipolar  supply voltages of ±4.5 V to ±16.5 V. In addition, the parts have  software-selectable output ranges of +5 V, +10 V, +10.8 V, ±5 V,  ±10 V, and ±10.8 V. Data is written to the AD5722/AD5732/  AD5752 in a 24-bit word format via a 3-wire serial interface.  The devices also offer an SDO pin to facilitate daisy-chaining  or readback.

The AD5722/AD5732/AD5752 incorporate a power-on reset  circuit to ensure that the DAC registers power up loaded with  0x0000. When powered on, the outputs are clamped to 0 V via  a low impedance path.

ARCHITECTURE

The DAC architecture consists of a string DAC followed by an  output amplifier. Figure 38 shows a block diagram of the DAC  architecture. The reference input is buffered before being  applied to the DAC.

REFIN

REF (+)

RESISTOR

DAC REGISTER VOUTX STRING

REF (–) CONFIGURABLE

OUTPUT

AMPLIFIER

The resistor string structure is shown in Figure 39. It is a string  of resistors, each of value R. The code loaded to the DAC register  determines the node on the string where the voltage is to be  tapped off and fed into the output amplifier. The voltage is  tapped off by closing one of the switches connecting the string  to the amplifier. Because it is a string of resistors, it is guaranteed  monotonic.

Output Amplifiers

The output amplifiers are capable of generating both unipolar  and bipolar output voltages. They are capable of driving a load  of 2 kΩ in parallel with 4000 pF to GND. The source and sink

capabilities of the output amplifiers can be seen in Figure 26.  The slew rate is 3.5 V/μs with a full-scale settling time of 10 μs.

Reference Buffers

The AD5722/AD5732/AD5752 require an external reference  source. The reference input has an input range of 2 V to 3 V,  with 2.5 V for specified performance. This input voltage is then  buffered before it is applied to the DAC cores.

POWER-UP SEQUENCE

Because the DAC output voltage is controlled by the voltage  monitor and control block (see Figure 42), it is important to  power the DVCC pin before applying any voltage to the AVDD and AVSS pins; otherwise, the G1 and G2 transmission gates are at

an undefined state. The ideal power-up sequence is in the

06467-006

GND

OUTPUT

RANGE CONTROL

Figure 38. DAC Architecture Block Diagram

REFIN

R

R

R TO OUTPUT

AMPLIFIER

R

R

06467-007

Figure 39. Resistor String Structure

following order: GND, SIG_GND, DAC_GND, DVCC, AVDD,  AVSS, and then the digital inputs. The relative order of powering  AVDD and AVSS is not important, provided that they are powered  up after DVCC.

SERIAL INTERFACE

The AD5722/AD5732/AD5752 are controlled over a versatile  3-wire serial interface that operates at clock rates up to 30 MHz.  It is compatible with SPI, QSPI™, MICROWIRE™, and DSP  standards.

Input Shift Register

The input shift register is 24 bits wide. Data is loaded into the  device MSB first as a 24-bit word under the control of a serial  clock input, SCLK. The input register consists of a read/write  bit, three register select bits, three DAC address bits, and 16 data

bits. The timing diagram for this operation is shown in Figure 2.

Rev. F | Page 18 of 31

Data Sheet AD5722/AD5732/AD5752

Standalone Operation

The serial interface works with both a continuous and noncon tinuous serial clock. A continuous SCLK source can be used  only if SYNC is held low for the correct number of clock cycles.  In gated clock mode, a burst clock containing the exact number  of clock cycles must be used, and SYNC must be taken high  after the final clock to latch the data. The first falling edge of  SYNC starts the write cycle. Exactly 24 falling clock edges must  be applied to SCLK before SYNC is brought high again. If  SYNC is brought high before the 24th falling SCLK edge, the  data written is invalid. If more than 24 falling SCLK edges are  applied before SYNC is brought high, the input data is also  invalid. The input register addressed is updated on the rising  edge of SYNC. For another serial transfer to take place, SYNC must be brought low again. After the end of the serial data  transfer, data is automatically transferred from the input shift  register to the addressed register.

When the data has been transferred into the chosen register of  the addressed DAC, all DAC registers and outputs can be  updated by taking LDAC low while SYNC is high.

AD5722/

Daisy-Chain Operation

For systems that contain several devices, the SDO pin can be  used to daisy-chain several devices together. Daisy-chain mode  can be useful in system diagnostics and in reducing the number  of serial interface lines. The first falling edge of SYNC starts the  write cycle. SCLK is continuously applied to the input shift  register when SYNC is low. If more than 24 clock pulses are  applied, the data ripples out of the shift register and appears on  the SDO line. This data is clocked out on the rising edge of  SCLK and is valid on the falling edge. By connecting the SDO  of the first device to the SDIN input of the next device in the  chain, a multidevice interface is constructed. Each device in the  system requires 24 clock pulses. Therefore, the total number of  clock cycles must equal 24 × N, where N is the total number of  AD5722/AD5732/AD5752 devices in the chain. When the serial  transfer to all devices is complete, SYNC is taken high. This  latches the input data in each device in the daisy chain and  prevents any further data from being clocked into the input shift  register. The serial clock can be a continuous or a gated clock.

A continuous SCLK source can only be used if SYNC is held  low for the correct number of clock cycles. In gated clock mode,  a burst clock containing the exact number of clock cycles must

68HC11*

MOSI

SCK

PC7

PC6

MISO

AD5732/ AD5752* SDIN

SCLK

SYNC

LDAC

SDO

SDIN

AD5722/ AD5732/ AD5752*

SCLK

SYNC

LDAC

SDO

SDIN

AD5722/ AD5732/ AD5752* SCLK

SYNC

LDAC

SDO

be used, and SYNC must be taken high after the final clock to  latch the data.

Readback Operation

Readback mode is invoked by setting the R/W bit = 1 in the  write operation to the serial input shift register. (If the SDO  output is disabled via the SDO disable bit in the control register,  it is automatically enabled for the duration of the read operation,  after which it is disabled again.) With R/W = 1, Bit A2 to Bit A0,  in association with Bit REG2 to Bit REG0, select the register to  be read. The remaining data bits in the write sequence are don’t  care bits. During the next SPI write, the data appearing on the SDO  output contains the data from the previously addressed register.  For a read of a single register, the NOP command can be used in  clocking out the data from the selected register on SDO. The  readback diagram in Figure 4 shows the readback sequence.  For example, to read back the DAC register of Channel A, the  following sequence should be implemented:

1. Write 0x800000 to the AD5722/AD5732/AD5752 input  register. This configures the part for read mode with the  DAC register of Channel A selected. Note that all the data  bits, DB15 to DB0, are don’t care bits.

2. Follow this with a second write, a NOP condition, 0x180000.

06467-008

*ADDITIONAL PINS OMITTED FOR CLARITY.

During this write, the data from the register is clocked out

Figure 40. Daisy Chaining the AD5722/AD5732/AD5752

on the SDO line.

Rev. F | Page 19 of 31

AD5722/AD5732/AD5752 Data Sheet

LOAD DAC (LDAC)

After data has been transferred into the input register of the  DACs, there are two ways to update the DAC registers and DAC  outputs. Depending on the status of both SYNC and LDAC, one  of two update modes is selected: individual DAC updating or  simultaneous updating of all DACs.

OUTPUT

AMPLIFIER

CONFIGURING THE AD5722/AD5732/AD5752 When the power supplies are applied to the AD5722/AD5732/  AD5752, the power-on reset circuit ensures that all registers  default to 0. This places all channels in power-down mode. The  DVCC should be brought high before any of the interface lines  are powered. If this is not done the first write to the device may  be ignored. The first communication to the AD5722/AD5732/  AD5752 should be to set the required output range on all

REFIN LDAC

SCLK SYNC SDIN

12-/14-/16-BIT DAC DAC REGISTER INPUT REGISTER

INTERFACE LOGIC

SDO

VOUTX 06467-009

channels (the default range is the 5 V unipolar range) by writing  to the output range select register. The user should then write to  the power control register to power on the required channels. To  program an output value on a channel, that channel must first

be powered up; any writes to a channel while it is in power-down  mode are ignored. The AD5722/AD5732/AD5752 operate with a  wide power supply range. It is important that the power supply  applied to the parts provide adequate headroom to support the  chosen output ranges.

TRANSFER FUNCTION

Figure 41. Simplified Diagram of Input Loading Circuitry for One DAC  Individual DAC Updating

In this mode, LDAC is held low while data is clocked into the  input shift register. The addressed DAC output is updated on  the rising edge of SYNC.

Simultaneous Updating of All DACs

In this mode, LDAC is held high while data is clocked into the  input shift register. All DAC outputs are asynchronously updated  by taking LDAC low after SYNC has been taken high. The  update now occurs on the falling edge of LDAC.

ASYNCHRONOUS CLEAR (CLR)

CLR is an active low clear that allows the outputs to be cleared  to either zero-scale code or midscale code. The clear code value is  user-selectable via the CLR select bit of the control register (see  the Control Register section). It is necessary to maintain CLR low  for a minimum amount of time to complete the operation (see  Figure 2). When the CLR signal is returned high, the output  remains at the cleared value until a new value is programmed.  The outputs cannot be updated with a new value while the CLR pin is low. A clear operation can also be performed via the clear  command in the control register.

Table 7 to Table 15 show the relationships of the ideal input  code to output voltage for the AD5752, AD5732, and AD5722,  respectively, for all output voltage ranges. For unipolar output  ranges, the data coding is straight binary. For bipolar output  ranges, the data coding is user selectable via the BIN/2sCOMP pin and can be either offset binary or twos complement.

For a unipolar output range, the output voltage expression is  given by

D VV Gain2

⎥⎦⎤ ⎢⎣⎡ ×= OUT REFIN N

For a bipolar output range, the output voltage expression is given by  VV Gain × −⎥⎦⎤ ⎢⎣⎡ ×=

D VGain

REFIN

OUT REFIN N

2 2

where:

D is the decimal equivalent of the code loaded to the DAC.  N is the bit resolution of the DAC.

VREFIN is the reference voltage applied at the REFIN pin.  Gain is an internal gain whose value depends on the output  range selected by the user, as shown in Table 6.

Table 6. Internal Gain Values

Output Range (V) Gain Value  +5 2

+10 4

+10.8 4.32

±5 4

±10 8

±10.8 8.64

Rev. F | Page 20 of 31

Data Sheet AD5722/AD5732/AD5752

Ideal Output Voltage to Input Code Relationship—AD5752

Table 7. Bipolar Output, Offset Binary Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (32,767/32,768) +2 × REFIN × (32,766/32,768) … +2 × REFIN × (1/32,768) 0 V −2 × REFIN × (1/32,768) … −2 × REFIN × (32,767/32,768) −2 × REFIN × (32,768/32,768) | +4 × REFIN × (32,767/32,768) +4 × REFIN × (32,766/32,768) … +4 × REFIN × (1/32,768) 0 V −4 × REFIN × (1/32,768) … −4 × REFIN × (32,767/32,768) −4 × REFIN × (32,768/32,768) |

MSB LSB ±10.8 V Output Range  1111 1111 1111 1111 +4.32 × REFIN × (32,767/32,768)  1111 1111 1111 1110 +4.32 × REFIN × (32,766/32,768)  … … … … …  1000 0000 0000 0001 +4.32 × REFIN × (1/32,768)  1000 0000 0000 0000 0 V  0111 1111 1111 1111 −4.32 × REFIN × (32,766/32,768)  … … … … …  0000 0000 0000 0001 −4.32 × REFIN × (32,767/32,768)  0000 0000 0000 0000 −4.32 × REFIN × (32,768/32,768)

Table 8. Bipolar Output, Twos Complement Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (32,767/32,768) +2 × REFIN × (32,766/32,768) … +2 × REFIN × (1/32,768) 0 V −2 × REFIN × (1/32,768) … −2 × REFIN × (32,767/32,768) −2 × REFIN × (32,768/32,768) | +4 × REFIN × (32,767/32,768) +4 × REFIN × (32,766/32,768) … +4 × REFIN × (1/32,768) 0 V −4 × REFIN × (1/32,768) … −4 × REFIN × (32,767/32,768) −4 × REFIN × (32,768/32,768) |

MSB LSB ±10.8 V Output Range  0111 1111 1111 1111 +4.32 × REFIN × (32,767/32,768)  0111 1111 1111 1110 +4.32 × REFIN × (32,766/32,768)  … … … … …  0000 0000 0000 0001 +4.32 × REFIN × (1/32,768)  0000 0000 0000 0000 0 V  1111 1111 1111 1111 −4.32 × REFIN × (1/32,768)  … … … … …  1000 0000 0000 0001 −4.32 × REFIN × (32,767/32,768)  1000 0000 0000 0000 −4.32 × REFIN × (32,768/32,768)

Table 9. Unipolar Output, Straight Binary Coding

Digital Input Analog Output

| +5 V Output Range | +10 V Output Range |
| --- | --- |
| +2 × REFIN × (65,535/65,536) +2 × REFIN × (65,534/65,536) … +2 × REFIN × (32,769/65,536) +2 × REFIN × (32,768/65,536) +2 × REFIN × (32,767/65,536) … +2 × REFIN × (1/65,536) 0 V | +4 × REFIN × (65,535/65,536) +4 × REFIN × (65,534/65,536) … +4 × REFIN × (32,769/65,536) +4 × REFIN × (32,768/65,536) +4 × REFIN × (32,767/65,536) … +4 × REFIN × (1/65,536) 0 V |

MSB LSB +10.8 V Output Range  1111 1111 1111 1111 +4.32 × REFIN × (65,535/65,536)  1111 1111 1111 1110 +4.32 × REFIN × (65,534/65,536)  … … … … …  1000 0000 0000 0001 +4.32 × REFIN × (32,769/65,536)  1000 0000 0000 0000 +4.32 × REFIN × (32,768/65,536)  0111 1111 1111 1111 +4.32 × REFIN × (32,767/65,536)  … … … … …  0000 0000 0000 0001 +4.32 × REFIN × (1/65,536)  0000 0000 0000 0000 0 V

Rev. F | Page 21 of 31

AD5722/AD5732/AD5752 Data Sheet

Ideal Output Voltage to Input Code Relationship—AD5732

Table 10. Bipolar Output, Offset Binary Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (8191/8192) +2 × REFIN × (8190/8192) … +2 × REFIN × (1/8192) 0 V −2 × REFIN × (1/8192) … −2 × REFIN × (8191/8192) −2 × REFIN × (8192/8192) | +4 × REFIN × (8191/8192) +4 × REFIN × (8190/8192) … +4 × REFIN × (1/8192) 0 V −4 × REFIN × (1/8192) … −4 × REFIN × (8191/8192) −4 × REFIN × (8192/8192) |

MSB LSB ±10.8 V Output Range  11 1111 1111 1111 +4.32 × REFIN × (8191/8192)  11 1111 1111 1110 +4.32 × REFIN × (8190/8192)  … … … … …  10 0000 0000 0001 +4.32 × REFIN × (1/8192)  10 0000 0000 0000 0 V  01 1111 1111 1111 −4.32 × REFIN × (1/8192)  … … … … …  00 0000 0000 0001 −4.32 × REFIN × (8191/8192)  00 0000 0000 0000 −4.32 × REFIN × (8192/8192)

Table 11. Bipolar Output, Twos Complement Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (8191/8192) +2 × REFIN × (8190/8192) … +2 × REFIN × (1/8192) 0 V −2 × REFIN × (1/8192) … −2 × REFIN × (8191/8192) −2 × REFIN × (8192/8192) | +4 × REFIN × (8191/8192) +4 × REFIN × (8190/8192) … +4 × REFIN × (1/8192) 0 V −4 × REFIN × (1/8192) … −4 × REFIN × (8191/8192) −4 × REFIN × (8192/8192) |

MSB LSB ±10.8 V Output Range  01 1111 1111 1111 +4.32 × REFIN × (8191/8192)  01 1111 1111 1110 +4.32 × REFIN × (8190/8192)  … … … … …  00 0000 0000 0001 +4.32 × REFIN × (1/8192)  00 0000 0000 0000 0 V  11 1111 1111 1111 −4.32 × REFIN × (1/8192)  … … … … …  10 0000 0000 0001 −4.32 × REFIN × (8191/8192)  10 0000 0000 0000 −4.32 × REFIN × (8192/8192)

Table 12. Unipolar Output, Straight Binary Coding

Digital Input Analog Output

| +5 V Output Range | +10 V Output Range |
| --- | --- |
| +2 × REFIN × (16,383/16,384) +2 × REFIN × (16,382/16,384) … +2 × REFIN × (8193/16,384) +2 × REFIN × (8192/16,384) +2 × REFIN × (8191/16,384) … +2 × REFIN × (1/16,384) 0 V | +4 × REFIN × (16,383/16,384) +4 × REFIN × (16,382/16,384) … +4 × REFIN × (8193/16,384) +4 × REFIN × (8192/16,384) +4 × REFIN × (8191/16,384) … +4 × REFIN × (1/16,384) 0 V |

MSB LSB +10.8 V Output Range  11 1111 1111 1111 +4.32 × REFIN × (16,383/16,384)  11 1111 1111 1110 +4.32 × REFIN × (16,382/16,384)  … … … … …  10 0000 0000 0001 +4.32 × REFIN × (8193/16,384)  10 0000 0000 0000 +4.32 × REFIN × (8192/16,384)  01 1111 1111 1111 +4.32 × REFIN × (8191/16,384)  … … … … …  00 0000 0000 0001 +4.32 × REFIN × (1/16,384)  00 0000 0000 0000 0 V

Rev. F | Page 22 of 31

Data Sheet AD5722/AD5732/AD5752

Ideal Output Voltage to Input Code Relationship—AD5722

Table 13. Bipolar Output, Offset Binary Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (2047/2048) +2 × REFIN × (2046/2048) … +2 × REFIN × (1/2048) 0 V −2 × REFIN × (1/2048) … −2 × REFIN × (2047/2048) −2 × REFIN × (2048/2048) | +4 × REFIN × (2047/2048) +4 × REFIN × (2046/2048) … +4 × REFIN × (1/2048) 0 V −4 × REFIN × (1/2048) … −4 × REFIN × (2047/2048) −4 × REFIN × (2048/2048) |

MSB LSB ±10.8 V Output Range  1111 1111 1111 +4.32 × REFIN × (2047/2048)  1111 1111 1110 +4.32 × REFIN × (2046/2048)  … … … …

1000 0000 0001 +4.32 × REFIN × (1/2048)  1000 0000 0000 0 V  0111 1111 1111 −4.32 × REFIN × (1/2048)  … … … …

0000 0000 0001 −4.32 × REFIN × (2047/2048)  0000 0000 0000 −4.32 × REFIN × (2048/2048)

Table 14. Bipolar Output, Twos Complement Coding

Digital Input Analog Output

| ±5 V Output Range | ±10 V Output Range |
| --- | --- |
| +2 × REFIN × (2047/2048) +2 × REFIN × (2046/2048) … +2 × REFIN × (1/2048) 0 V −2 × REFIN × (1/2048) … −2 × REFIN × (2047/2048) −2 × REFIN × (2048/2048) | +4 × REFIN × (2047/2048) +4 × REFIN × (2046/2048) … +4 × REFIN × (1/2048) 0 V −4 × REFIN × (1/2048) … −4 × REFIN × (2047/2048) −4 × REFIN × (2048/2048) |

MSB LSB ±10.8 V Output Range  0111 1111 1111 +4.32 × REFIN × (2047/2048)  0111 1111 1110 +4.32 × REFIN × (2046/2048)  … … … …

0000 0000 0001 +4.32 × REFIN × (1/2048)  0000 0000 0000 0 V

1111 1111 1111 −4.32 × REFIN × (1/2048)  … … … …

1000 0000 0001 −4.32 × REFIN × (2047/2048)  1000 0000 0000 −4.32 × REFIN × (2048/2048)

Table 15. Unipolar Output, Straight Binary Coding

Digital Input Analog Output

| +5 V Output Range | +10 V Output Range |
| --- | --- |
| +2 × REFIN × (4095/4096) +2 × REFIN × (4094/4096) … +2 × REFIN × (2049/4096) +2 × REFIN × (2048/4096) +2 × REFIN × (2047/4096) … +2 × REFIN × (1/4096) 0 V | +4 × REFIN × (4095/4096) +4 × REFIN × (4094/4096) … +4 × REFIN × (2049/4096) +4 × REFIN × (2048/4096) +4 × REFIN × (2047/4096) … +4 × REFIN × (1/4096) 0 V |

MSB LSB +10.8 V Output Range  1111 1111 1111 +4.32 × REFIN × (4095/4096)  1111 1111 1110 +4.32 × REFIN × (4094/4096)  … … … …

1000 0000 0001 +4.32 × REFIN × (2049/4096)  1000 0000 0000 +4.32 × REFIN × (2048/4096)  0111 1111 1111 +4.32 × REFIN × (2047/4096)  … … … …

0000 0000 0001 +4.32 × REFIN × (1/4096)  0000 0000 0000 0 V

Rev. F | Page 23 of 31

AD5722/AD5732/AD5752 Data Sheet

INPUT SHIFT REGISTER

The input shift register is 24 bits wide and consists of a read/write bit (R/W), a reserved bit (zero) that must always be set to 0, three  register select bits (REG0, REG1, REG2), three DAC address bits (A2, A1, A0), and 16 data bits (data). The register data is clocked in MSB  first on the SDIN pin. Table 16 shows the register format, and Table 17 describes the function of each bit in the register. All registers are  read/write registers.

Table 16. Input Register Format

MSB LSB

| DB23 | DB22 | DB21 | DB20 | DB19 | DB18 | DB17 | DB16 | DB15 to DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | Data |

Table 17. Input Register Bit Functions

Bit Mnemonic Description

R/W Indicates a read from or a write to the addressed register.

REG2, REG1, REG0 Used in association with the address bits to determine if a write operation is to the DAC register, the output range  select register, the power control register, or the control register.

| REG2 | REG1 | REG0 |
| --- | --- | --- |
| 0 0 0 0 | 0 0 1 1 | 0 1 0 1 |

Function

DAC register  Output range select register

Power control register

Control register  A2, A1, A0 These DAC address bits are used to decode the DAC channels.

| A2 | A1 | A0 |
| --- | --- | --- |
| 0 0 1 | 0 1 0 | 0 0 0 |

Channel Address

DAC A   DAC B   Both DACs  Data Data bits.

Rev. F | Page 24 of 31

Data Sheet AD5722/AD5732/AD5752

DAC REGISTER

The DAC register is addressed by setting the three REG bits to 000. The DAC address bits select the DAC channel in which the data  transfer is to take place (see Table 17). The data bits are in positions DB15 to DB0 for the AD5752 (see Table 18), DB15 to DB2 for the  AD5732 (see Table 19), and DB15 to DB4 for the AD5722 (see Table 20).

Table 18. Programming the AD5752 DAC Register

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | DAC address |  |  | 16-bit DAC data |

Table 19. Programming the AD5732 DAC Register

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB2 | DB1 | DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | DAC address |  |  | 14-bit DAC data | X | X |

Table 20. Programming the AD5722 DAC Register

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB4 | DB3 | DB2 | DB1 | DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | DAC address |  |  | 12-bit DAC data | X | X | X | X |

OUTPUT RANGE SELECT REGISTER

The output range select register is addressed by setting the three REG bits to 001. The DAC address bits select the DAC channel, and the  range bits (R2, R1, R0) select the required output range (see Table 21 and Table 22).

Table 21. Programming the Required Output Range

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB3 | DB2 | DB1 | DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | DAC address |  |  | Don’t care | R2 | R1 | R0 |

Table 22. Output Range Options

| R1 | R0 |
| --- | --- |
| 0 0 1 1 0 0 | 0 1 0 1 0 1 |

R2 Output Range (V)

0 +5

0 +10

0 +10.8

0 ±5

1 ±10

1 ±10.8

Rev. F | Page 25 of 31

AD5722/AD5732/AD5752 Data Sheet

CONTROL REGISTER

The control register is addressed by setting the three REG bits to 011. The value written to the address and data bits determines the  control function selected. The control register options are shown in Table 23 and Table 24.

Table 23. Programming the Control Register

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB4 | DB3 | DB2 | DB1 | DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | NOP, data = don’t care |  |  |  |  |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | Don’t care | TSD enable | Clamp enable | CLR select | SDO disable |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | Clear, data = don’t care |  |  |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | Load, data = don’t care |  |  |  |  |

Table 24. Explanation of Control Register Options

Option Description

NOP No operation instruction used in readback operations.

Clear Addressing this function sets the DAC registers to the clear code and updates the outputs.

Load Addressing this function updates the DAC registers and, consequently, the DAC outputs.

SDO Disable Set by the user to disable the SDO output. Cleared by the user to enable the SDO output (default).  CLR Select See Table 25 for a description of the CLR select operation.

Clamp Enable Set by the user to enable the current-limit clamp. The channel does not power down upon detection of an overcurrent; the  current is clamped at 20 mA (default).

Cleared by the user to disable the current-limit clamp. The channel powers down upon detection of an overcurrent.  TSD Enable Set by the user to enable the thermal shutdown feature. Cleared by the user to disable the thermal shutdown feature (default).

Table 25. CLR Select Options

Output CLR Value

Unipolar Output Range

0 V Midscale

CLR Select Setting Bipolar Output Range  0 0 V

1 Negative full-scale

POWER CONTROL REGISTER

The power control register is addressed by setting the three REG bits to 010. This register allows the user to control and determine the  power and thermal status of the AD5722/AD5732/AD5752. The power control register options are shown in Table 26 and Table 27.

Table 26. Programming the Power Control Register

MSB LSB

| R/W | Zero | REG2 | REG1 | REG0 | A2 | A1 | A0 | DB15 to DB11 | DB10 | DB9 | DB8 | DB7 | DB6 | DB5 | DB4 | DB3 | DB2 | DB1 | DB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | X | 0 | OCB | X | OCA | X | TSD | X | X | PUB | X | PUA |

Table 27. Power Control Register Functions

Option Description

PUA DAC A power-up. When set, this bit places DAC A in normal operating mode. When cleared, this bit places DAC A in power-down  mode (default). After setting this bit to power DAC A, a power-up time of 10 μs is required. During this power-up time, the DAC register should not be loaded to the DAC output (see the Load DAC (LDAC) section). If the clamp enable bit of the control register  is cleared, DAC A powers down automatically on detection of an overcurrent, and PUA is cleared to reflect this.

PUB DAC B power-up. When set, this bit places DAC B in normal operating mode. When cleared, this bit places DAC B in power-down  mode (default). After setting this bit to power DAC B, a power-up time of 10 μs is required. During this power-up time, the DAC register should not be loaded to the DAC output (see the Load DAC (LDAC) section). If the clamp enable bit of the control register  is cleared, DAC B powers down automatically on detection of an overcurrent, and PUB is cleared to reflect this.

TSD Thermal shutdown alert (read-only bit). In the event of an overtemperature situation, both DACs are powered down and this bit is set.  OCA DAC A overcurrent alert (read-only bit). In the event of an overcurrent situation on DAC A, this bit is set.  OCB DAC B overcurrent alert (read-only bit). In the event of an overcurrent situation on DAC B, this bit is set.

Rev. F | Page 26 of 31

Data Sheet AD5722/AD5732/AD5752

DESIGN FEATURES

ANALOG OUTPUT CONTROL

In many industrial process control applications, it is vital that  the output voltage be controlled during power-up. When the  supply voltages change during power-up, the VOUTx pins are  clamped to 0 V via a low impedance path (approximately 4 kΩ).  To prevent the output amplifiers from being shorted to 0 V  during this time, Transmission Gate G1 is also opened (see  Figure 42). These conditions are maintained until the analog  power supplies have stabilized and a valid word is written to a  DAC register. At this time, G2 opens and G1 closes.

VOLTAGE

MONITOR

AND

CONTROL

G1

VOUTA

G2

06467-010

Figure 42. Analog Output Control Circuitry

POWER-DOWN MODE

Each DAC channel of the AD5722/AD5732/AD5752 can be  individually powered down. By default, all channels are in  power-down mode. The power status is controlled by the power  control register (see Table 26 and Table 27 for details). When a  channel is in power-down mode, its output pin is clamped to  ground through a resistance of approximately 4 kΩ, and the  output of the amplifier is disconnected from the output pin.

OVERCURRENT PROTECTION

Each DAC channel of the AD5722/AD5732/AD5752 incorporates  individual overcurrent protection. The user has two options for  the configuration of the overcurrent protection: constant current  clamp or automatic channel power-down. The configuration of  the overcurrent protection is selected via the clamp enable bit in  the control register.

Constant Current Clamp (Clamp Enable = 1)  If a short circuit occurs in this configuration, the current is  clamped at 20 mA. This event is signaled to the user by the  setting of the appropriate overcurrent (OCX) bit in the power  control register. Upon removal of the short-circuit fault, the  OCX bit is cleared.

Automatic Channel Power-Down (Clamp Enable = 0)  If a short circuit occurs in this configuration, the shorted channel  powers down and its output is clamped to ground via a resistance  of approximately 4 kΩ. At this time, the output of the amplifier  is disconnected from the output pin. The short-circuit event is  signaled to the user via the overcurrent (OCX) bits, and the  power-up (PUX) bits indicate which DACs have powered down.  After the fault is rectified, the channels can be powered up again  by setting the PUX bits.

THERMAL SHUTDOWN

The AD5722/AD5732/AD5752 incorporate a thermal shutdown  feature that automatically shuts down the device if the core  temperature exceeds approximately 150°C. The thermal shutdown  feature is disabled by default and can be enabled via the TSD  enable bit of the control register. In the event of a thermal  shutdown, the TSD bit of the power control register is set.

Rev. F | Page 27 of 31

AD5722/AD5732/AD5752 Data Sheet

APPLICATIONS INFORMATION

+5 V/±5 V OPERATION

When operating from a single +5 V supply or a dual ±5 V supply,  an output range of +5 V or ±5 V is not achievable because suffi cient headroom for the output amplifier is not available. In this  situation, a reduced reference voltage can be used. For example,  a 2 V reference voltage produces an output range of +4 V or ±4 V,

and the 1 V of headroom is more than enough for full operation.  A standard value voltage reference of 2.048 V can be used to  produce output ranges of +4.096 V and ±4.096 V.

ALTERNATIVE POWER-UP SEQUENCE SUPPORT  There may be cases where it is not possible to use the  recommended power-up sequence, and in those instances an  external circuit shown in Figure 43 is recommended to be used.  The circuit shown in Figure 43 ensures that the digital block is  powered up first, prior to the analog block, by using a load  switch circuit. This circuit targets applications for which either  AVDD or AVSS or both supplies power up before DVCC.

Consider the following design rules when choosing the  component values for the AVDD delay circuit.

∙ R1 ensures that the Q1 gate to source voltage is zero when  DVCC is in an open state. R1 also prevents false turn on of  Q1. However, if DVCC is permanently connected to the  source, R1 can be removed to conserve power.

∙ Select Q1 (N-channel MOSFET) with a VGS threshold that  is much lower than the minimum operating DVCC and a  VDS rating much lower than the maximum operating AVDD.

∙ C1, R2, and R3 are the main components that dictates the  delay from DVCC enable to AVDD. Adjust the values  according for the desired delay. Choose R2 and R3 values  that ensure Q2 turn on.

Figure 44 shows an example of the analog supplies powering up  before the digital supply. The circuit delays the AVDD power up  until after DVCC, as shown by the AVDD (delayed) line.

AVDD

AVSS

t (sec)

DVCC

AVDD (DELAYED)

06467-144

Figure 44. Delayed Power Supplies Sequence Example

LAYOUT GUIDELINES

In any circuit where accuracy is important, careful consideration  of the power supply and ground return layout helps to ensure  the rated performance. The printed circuit board on which the  AD5722/AD5732/AD5752 are mounted should be designed so  that the analog and digital sections are separated and confined  to certain areas of the board. If the AD5722/AD5732/AD5752 are in a system where multiple devices require an AGND-to DGND connection, the connection should be made at one  point only. The star ground point should be established as close  as possible to the device.

⎢⎢⎣⎡⎟⎟⎠⎞

⎥⎥⎦⎤

⎜⎜⎝⎛ −= −×EQ V

The AD5722/AD5732/AD5752 should have ample supply bypass ing of a 10 μF capacitor in parallel with a 0.1 μF capacitor on

GS

t RRC 1ln)||()sec( 231

DELAY V ⎜⎜⎝⎛+ =23

where ⎟⎟⎠⎞

each supply located as close to the package as possible, ideally  right up against the device. The 10 μF capacitor is the tantalum

EQ AVV DD

R

bead type. The 0.1 μF capacitor should have low effective series

3

RR

∙ Q2 (P-channel MOSFET) acts as a switch that allows the flow  of current from VIN to AVDD; therefore, choosing a MOSFET  with very low RDSON is necessary to minimize losses during  operation. Other parameters such as maximum VDS rating,  maximum drain to source current rating, VGS threshold

voltage, and maximum gate to source voltage rating must  also be taken into consideration when choosing Q2.

VIN AVDD

resistance (ESR) and low effective series inductance (ESI) such  as the common ceramic types, which provide a low impedance  path to ground at high frequencies to handle transient currents  due to internal logic switching.

The power supply lines of the AD5722/AD5732/AD5752 should  use as large a trace as possible to provide low impedance paths  and reduce the effects of glitches on the power supply line. Fast  switching signals, such as a data clock, should be shielded with  digital ground to avoid radiating noise to other parts of the

+

C1

R3

Q2

board, and they should never be run near the reference inputs.

DVCC

LOAD SWITCH SECTION

CONTROL

SECTION

R1

R2

Q1

A ground line routed between the SDIN and SCLK lines helps

reduce crosstalk between them (this is not required on a

multilayer board that has a separate ground plane, but separating

06467-143

the lines does help). It is essential to minimize noise on the

Figure 43. Load Switch Control Circuit

Rev. F | Page 28 of 31

Data Sheet AD5722/AD5732/AD5752

REFIN line because any unwanted signals can couple through  to the DAC outputs.

Avoid crossover of digital and analog signals. Traces on  opposite sides of the board should run at right angles to each  other. This reduces the effects of feedthrough on the board. A  microstrip technique is by far the best method, but it is not  always possible with a double-sided board. In this technique,  the component side of the board is dedicated to a ground plane,  and signal traces are placed on the solder side.

GALVANICALLY ISOLATED INTERFACE  In many process control applications, it is necessary to provide  an isolation barrier between the controller and the unit being  controlled to protect and isolate the controlling circuitry from  any hazardous common-mode voltages that may occur. The  iCoupler® family of products from Analog Devices, Inc., provides  voltage isolation in excess of 2.5 kV. The serial loading structure  of the AD5722/AD5732/AD5752 makes them ideal for isolated  interfaces because the number of interface lines is kept to a  minimum. Figure 45 shows a 4-channel isolated interface to the  AD5722/AD5732/AD5752 using an ADuM1400. For further  information, visit www.analog.com/iCouplers.

MICROCONTROLLER ADuM1400*

∙ The temperature coefficient of a reference output voltage  affects INL, DNL, and TUE. A reference with a tight  temperature coefficient specification should be chosen to  reduce the dependence of the DAC output voltage on  ambient conditions.

∙ Long-term drift is a measure of how much the reference  output voltage drifts over time. A reference with a tight  long-term drift specification ensures that the overall  solution remains relatively stable over its entire lifetime.

∙ Reference output voltage noise needs to be considered in  high accuracy applications that have relatively low noise  budgets. It is important to choose a reference with as low  an output noise voltage as practical for the required system  resolution. Precision voltage references such as the ADR431 (XFET® design) produce low output noise in the 0.1 Hz to  10 Hz range. However, as the circuit bandwidth increases,  filtering the output of the reference may be required to  minimize the output noise.

MICROPROCESSOR INTERFACING

Microprocessor interfacing to the AD5722/AD5732/AD5752 is  via a serial bus that uses a standard protocol compatible with  microcontrollers and DSP processors. The communications

SERIAL CLOCK OUT SERIAL DATA OUT

ENCODE DECODE VIA

VIB

ENCODE DECODE

VOA VOB

TO SCLK TO SDIN

channel is a 3-wire (minimum) interface consisting of a clock  signal, a data signal, and a synchronization signal. The AD5722/  AD5732/AD5752 require a 24-bit data-word with data valid on

SYNC OUT

VIC

ENCODE DECODE

VOC

TO SYNC

the falling edge of SCLK.

CONTROL OUT

VID

VOD ENCODE DECODE

TO LDAC

For all interfaces, the DAC output update can be initiated

06467-011

*ADDITIONAL PINS OMITTED FOR CLARITY.

Figure 45. Isolated Interface

VOLTAGE REFERENCE SELECTION

automatically when all the data is clocked in, or it can be  performed under the control of LDAC. The contents of the  registers can be read using the readback function.

AD5722/AD5732/AD5752 to Blackfin® DSP Interface

To achieve optimum performance from the AD5722/AD5732/  AD5752 over their full operating temperature range, a precision  voltage reference must be used. Thought should be given to the  selection of a precision voltage reference. The voltage applied to  the reference inputs is used to provide a buffered positive and  negative reference for the DAC cores. Therefore, any error in

Figure 46 shows how the AD5722/AD5732/AD5752 can be  interfaced to the Analog Devices Blackfin DSP. The Blackfin has  an integrated SPI port that can be connected directly to the SPI  pins of the AD5722/AD5732/AD5752 and the programmable  I/O pins that can be used to set the state of a digital input such  as the LDAC pin.

the voltage reference is reflected in the outputs of the device.

There are four possible sources of error to consider when  choosing a voltage reference for high accuracy applications:  initial accuracy, temperature coefficient of the output voltage,  long-term drift, and output voltage noise.

SPISELx

SCK

MOSI

ADSP-BF531

SYNC

SCLK

SDIN

AD5722/ AD5732/ AD5752

∙ Initial accuracy error on the output voltage of an external  reference can lead to a full-scale error in the DAC. To  minimize these errors, a reference with low initial accuracy  error specification is preferred. Choosing a reference with  an output trim adjustment, such as the ADR421, allows a  system designer to trim out system errors by setting the  reference voltage to a voltage other than the nominal. The  trim adjustment can also be used to trim out temperature induced errors.

PF10 LDAC

06467-012

Figure 46. AD5722/AD5732/AD5752 to Blackfin Interface

Rev. F | Page 29 of 31

AD5722/AD5732/AD5752 Data Sheet

Table 28. Some Precision References Recommended for Use with the AD5722/AD5732/AD5752

| Initial Accuracy (mV Max) | Long-Term Drift (ppm Typ) | Temp Drift (ppm/°C Max) |
| --- | --- | --- |
| ±1 ±1 ±2.5 ±2 ±1 | 40 50 50 50 20 | 3 3 3 8 3 |

Part No. 0.1 Hz to 10 Hz Noise (μV p-p Typ)  ADR431 3.5

ADR421 1.75  ADR03 6

ADR291 8

AD780 4

Rev. F | Page 30 of 31

Data Sheet AD5722/AD5732/AD5752

OUTLINE DIMENSIONS

7.90

7.80

7.70

24 13

1 12 TOP VIEW

4.50

4.40

4.30

6.40 BSC

5.02

5.00

4.95

EXPOSED PAD

(Pins Up)

BOTTOM VIEW

3.25

3.20

3.15

FOR PROPER CONNECTION OF

1.20 MAX 1.05 1.00

8°

0.80

0°

THE EXPOSED PAD, REFER TO THE PIN CONFIGURATION AND FUNCTION DESCRIPTIONS SECTION OF THIS DATA SHEET.

0.15

0.65

0.30

0.20

0.75

0.05

0.19 SEATING

0.09

PLANE

0.10 COPLANARITY

BSC

0.60

0.45

061708-A

COMPLIANT TO JEDEC STANDARDS MO-153-ADT

Figure 47. 24-Lead Thin Shrink Small Outline Package, Exposed Pad [TSSOP_EP]

(RE-24)

Dimensions shown in millimeters

ORDERING GUIDE

| Resolution (Bits) | Temperature Range | INL | TUE (% FSR) | Package Description |
| --- | --- | --- | --- | --- |
| 12 12 | −40°C to +85°C −40°C to +85°C | ±1 LSB ±1 LSB | ±0.3 ±0.3 | 24-Lead TSSOP_EP 24-Lead TSSOP_EP |
| 14 14 | −40°C to +85°C −40°C to +85°C | ±4 LSB ±4 LSB | ±0.3 ±0.3 | 24-Lead TSSOP_EP 24-Lead TSSOP_EP |
| 16 16 | −40°C to +85°C −40°C to +85°C | ±16 LSB ±16 LSB | ±0.3 ±0.3 | 24-Lead TSSOP_EP 24-Lead TSSOP_EP |

Model1 Package Option  AD5722AREZ RE-24  AD5722AREZ-REEL7 RE-24  AD5732AREZ RE-24  AD5732AREZ-REEL7 RE-24  AD5752AREZ RE-24  AD5752AREZ-REEL7 RE-24

1 Z = RoHS Compliant Part.

©2008–2017 Analog Devices, Inc. All rights reserved. Trademarks and

registered trademarks are the property of their respective owners.

D06467-0-2/17(F)

Rev. F | Page 31 of 31